Zamboni General Operation Check-list

Prep

- Use large black hose to fill hot water tank on Zamboni (lever on wall).

- Use green hose to fill cold water rinse tank if not completely full.

- Inspect both propane tanks to ensure they are not empty. Turn dial on tanks to open/on for both.

- Replace auger covers (3) in slots on conditioner.

- Ensure rug is in lowered position.

- Once hot water tank is full, place black screw top lid on.

- Open Garage door.

- Open both rink doors.

- Ensure path is clear and no skaters remaining on the ice.

- Turn on Zamboni.

- Lift hopper slightly, with hand on yellow support bar.  Remove yellow support bar, place on tray with handle locked in, and then lower hopper.

- Increase RPM’s to max (toggle button to rabbit picture).

- Ensure conditioner is raised all the way.

- Exit Zamboni Garage onto ice slowly.

Resurfacing

- Once on the ice completely, lower conditioner while driving forward.

- Start auger (right side lever)

- Flip side wall brush to on.

- Turn hot water on.

- Turn cold water on.

- Turn cold water rinse impeller on.

- First lap around rink boards. Be careful not to run front of hopper into doorways nor conditioner into open Zamboni door boards.

- Flip switch to turn off sweeper arm after board lap.

- Next lap around exterior one Zamboni width off boards (driver side along water line).

- Pump the plunger to clear auger chute once a lap.

- When completing second lap (Zamboni end), turn up the middle and then turn right at the concessions end (clockwise).

- Turn down hot water when near far sides of ice.

Exiting the Ice

- Turn off cold water rinse switch.

- Turn off cold water on final turn.

- Turn down hot water to nearly off.

- When lined up straight to get off the ice, turn off auger.

- When 3 feet from the edge of the ice, pull up the conditioner all the way while maintaining forward motion.

- Pull Zamboni into garage.

- Shovel excess slush/snow off ice and into drain.

- Drill holes for pins.

- Close Zamboni rink side doors- slider and 2 pins.

Finishing

- Pull up the rug to the higher setting.

- Use garage door opener on Zamboni to exit building out back. Watch cement ledge. Find area to dump hopper.

- Raise hopper to dump snow.

- Lower hopper back to lowest position. Return to garage area.

- Turn on/off wheel rinse when entering/parking.

- Enter Zamboni garage.

- Raise Hopper and insert yellow bar.

- Turn down RMP’s

- Turn off Zamboni

- Check level on both gas tanks. If a tank is empty, replace with full tank. (New tanks located outside – full tanks are top side facing out, empty are bottom side facing out).

- Use black hose to rinse auger chute.

- Remove auger covers (3) and rinse conditioner with black hot water hose.